# Comprehensive Technical Report: Smart Bus Tracking and Seat Counting System Implementation

## Executive Summary

This detailed technical report outlines the architectural design, engineering specifications, and execution strategy for a comprehensive Smart Bus Tracking and Seat Counting System. Designed for deployment in intelligent transportation networks, the system leverages the ESP32-S3 microcontroller for edge computing, NavIC (Navigation with Indian Constellation) for geolocational sovereignty and precision, GSM/LTE modules for telemetry, and OpenCV combined with deep learning for passenger analytics.

The report addresses the critical need for real-time fleet visibility and automated capacity management. By processing visual data at the edge and utilizing high-precision regional satellite navigation, the proposed solution minimizes data transmission costs while maximizing operational insight. This document serves as a definitive guide for engineering teams, covering component selection, circuit topology, firmware logic, and backend integration.

## 1. Introduction and Strategic Context

### 1.1 The Imperative for Intelligent Transportation Systems (ITS)

Modern public transit systems face a dual challenge: optimizing fleet utilization while ensuring passenger comfort and reliability. Traditional tracking systems, reliant solely on GPS and basic GPRS telemetry, suffer from signal degradation in dense urban environments ("urban canyons") and fail to provide granular data on vehicle occupancy. The result is inefficiencies in route planning, with some buses running empty while others face overcrowding.

The integration of Edge AI and Regional Navigation Systems represents a paradigm shift. Moving computational loads from centralized servers to the vehicle (the "edge") reduces latency and operational costs associated with high-bandwidth video streaming. Simultaneously, shifting from global GPS to regional constellations like NavIC ensures higher signal availability and positional accuracy within specific geographics, such as the Indian subcontinent, complying with emerging governmental mandates for indigenous technology adoption.

### 1.2 Project Scope and Objectives

The objective of this project is to engineer a robust, embedded hardware solution that performs two simultaneous functions:

- High-Precision Tracking: utilizing the NavIC constellation to provide reliable latitude, longitude, and velocity data.

- Automated Seat Counting: Utilizing computer vision (OpenCV/AI) to analyze interior camera feeds, determine occupancy, and transmit simplified metadata to the cloud.

The system is built upon the ESP32-S3 platform, chosen for its dual-core architecture and specific vector instructions that accelerate neural network processing. Connectivity is managed via SIM7600 (4G LTE) modules to ensure low-latency communication, with SIM800L (2G) considered as a legacy fallback.

## 2. System Architecture and Design Philosophy

The system architecture is predicated on the principle of Distributed Intelligence. Rather than treating the bus unit as a "dumb" data pipe that streams video to a server for processing, the onboard unit functions as an intelligent node.

### 2.1 Architectural Block Diagram

The hardware ecosystem is divided into four distinct subsystems, interconnected via standard serial and parallel protocols.

- Core Processing Unit (CPU): The ESP32-S3 SoC. This unit acts as the orchestrator, managing the RTOS (Real-Time Operating System) scheduler, interfacing with peripherals, and executing the computer vision inference engine.

- Visual Acquisition Subsystem: An OV2640 or OV5640 camera module connected via a parallel DVP (Digital Video Port) interface. This subsystem captures the raw visual data of the bus interior.

- Positioning Subsystem: A NavIC-compatible GNSS module (e.g., Allystar GEM1205). This module continuously parses satellite signals on the L1 and L5 bands to resolve the vehicle's geospatial state.

- Telematics and Communication Subsystem: A SIM7600 LTE Modem. This unit handles the TCP/IP stack, managing MQTT (Message Queuing Telemetry Transport) sessions for telemetry upload and HTTP/FTP sessions for image transfer.

### 2.2 Data Flow Pipeline

The operational logic follows a cyclical pipeline designed to maximize throughput and minimize power consumption.

- Acquisition Phase: The RTOS triggers a camera capture event. The OV2640 flushes its buffer and transmits a JPEG-compressed or raw RGB565 frame to the ESP32-S3's PSRAM (Pseudo-Static RAM). Simultaneously, the UART interrupt service routine collects NMEA sentences from the NavIC module.

- Processing Phase:

  - Vision: The ESP32-S3 decompresses the image (if JPEG) or resizes the raw matrix using OpenCV primitives optimized for the XTensa LX7 core. It then passes the normalized matrix to a TensorFlow Lite Micro interpreter or a custom OpenCV-based blob detection algorithm to count passengers.

  - Location: The NMEA parser filters the stream for $GIGGA or $GNRMC sentences, extracting coordinates and validating the Horizontal Dilution of Precision (HDOP).

- Fusion and Transmission Phase: The passenger count integer is fused with the timestamp and location. This lightweight JSON payload (typically < 200 bytes) is serialized and pushed to the GSM modem's transmission queue.

- Persistence Phase: In the event of network failure, data is written to a local ring buffer on an SD card or SPIFFS (SPI Flash File System) to prevent data loss.

## 3. Hardware Specifications and Component Selection

The selection of hardware components is governed by the rigorous demands of the automotive environment: voltage fluctuations, vibration, thermal extremes, and the need for long-term reliability.

### 3.1 Microcontroller: ESP32-S3

The transition from the classic ESP32 to the ESP32-S3 is non-negotiable for this application due to specific architectural enhancements required for computer vision.

| Feature | Standard ESP32 | ESP32-S3 | Implications for Project |
| --- | --- | --- | --- |
| Processor Core | Xtensa LX6 (Dual) | Xtensa LX7 (Dual) | LX7 includes vector instructions for AI acceleration. |
| AI Performance | Basic | Enhanced | S3 executes neural network layers ~3x faster. |
| PSRAM Support | SPI (4MB limit typical) | Octal SPI (Up to 32MB) | Essential for high-res image buffering and model storage. |
| Camera Interface | I2S / Parallel | Dedicated LCD/CAM | Higher frame rates and lower CPU overhead during capture. |
| GPIO Count | 34 (many input only) | 45 (programmable) | Allows connecting Camera + SD Card + GSM + GPS simultaneously. |

Selection: The Seeed Studio XIAO ESP32S3 Sense is highlighted as a prime candidate due to its integrated camera connector, digital microphone (optional for noise monitoring), and compact form factor. Alternatively, custom PCBs should utilize the ESP32-S3-WROOM-1-N8R8 module (8MB Flash / 8MB PSRAM) to ensure sufficient heap memory for image manipulation.

### 3.2 NavIC GNSS Module Selection

Standard GPS modules (e.g., u-blox NEO-6M) are single-frequency (L1) devices tuned for the US GPS constellation. To leverage NavIC (IRNSS), the module must support the L5 frequency band (1176.45 MHz) and the specific PRN codes of the Indian constellation.

- Primary Recommendation: Allystar GEM1205

  - Constellation Support: GPS, GLONASS, Galileo, BeiDou, QZSS, and NavIC (IRNSS).

  - Bands: L1 & L5 Dual-band. Dual-band support is critical for correcting ionospheric errors, significantly improving accuracy in urban environments compared to single-band L1 receivers.

  - Sensitivity: Tracking -162 dBm.

  - Update Rate: 1Hz to 10Hz (1Hz is sufficient for bus tracking).

- Alternative: Quectel L89 R2.0

  - A widely available module with integrated NavIC support and reliable NMEA output. It often includes an integrated patch antenna, simplifying deployment.

Integration Note: NavIC modules output NMEA sentences with the Talker ID $GI (e.g., $GIGGA). Firmware parsers must be explicitly configured to recognize this ID, as generic libraries often default to filtering for $GP.

### 3.3 Cellular Communication: SIM7600 (LTE) vs. SIM800L (2G)

While the SIM800L is a popular low-cost option for hobbyist trackers, it is deemed unsuitable for a commercial-grade Smart Bus system.

- Obsolescence Risk: 2G networks are being sunset globally. Investing in 2G hardware creates immediate technical debt.

- Bandwidth Constraints: The upload speed of GPRS (approx. 50-80 kbps) is insufficient for uploading validation images. If the seat counting algorithm flags an anomaly (e.g., "Camera Blocked"), the operator needs to request a snapshot. LTE CAT-4 (SIM7600) provides the necessary bandwidth (Mbps) for this.

- Latency: LTE offers significantly lower latency for MQTT keep-alive packets, ensuring the bus's "Online" status is accurate on the dashboard.

Selection: SIM7600G-H (Global Band) or SIM7600E (Europe/Asia Band) is selected. It interfaces via UART and supports a rich set of AT commands for HTTP(S), MQTT, and FTP.

### 3.4 Camera Sensor: OV2640

The OV2640 remains the standard for ESP32-based vision due to its hardware JPEG encoder.

- Resolution: 2 Megapixels (1600x1200). For seat counting, resolution is typically downscaled to QVGA (320x240) or VGA (640x480) to balance detail with processing speed.

- Optics: Standard lenses have a Field of View (FOV) of approx. 60°. For a bus interior, a 120° or 160° Wide-Angle Lens is mandatory to capture the entire seating aisle from a single mounting point.

## 4. Electrical Engineering and Power Management

Designing the power supply for a vehicle tracker is complex due to the "dirty" power found in automotive systems. Voltage spikes from the alternator (load dumps) can reach 60V, destroying sensitive 3.3V electronics.

### 4.1 Power Supply Unit (PSU) Design

The system requires a multi-stage regulation topology. Linear regulators (LDOs) are inefficient for the primary stage and will overheat.

#### Stage 1: High-Voltage Buck Conversion

- Component: LM2596-5.0 or MP1584 Step-Down Converter.

- Input: 9V - 36V DC (covering 12V and 24V vehicle batteries).

- Output: 5.0V regulated rail.

- Current Capacity: Must support at least 3.0A. The SIM7600 modem can draw transmission bursts of up to 2A. The ESP32-S3 and camera add another ~300-500mA. A 3A regulator provides necessary headroom.

- Protection: A TVS (Transient Voltage Suppressor) diode and a resettable fuse (PTC) must be placed at the input to protect against voltage spikes and short circuits.

#### Stage 2: Modem Power Buffering

- Challenge: GSM burst transmission causes rapid voltage drops (droop) on the power rail. If the voltage drops below 3.3V, the ESP32 will brownout and reset.

- Solution: A large bulk capacitor (1000µF to 2200µF Low-ESR Electrolytic) is placed in parallel with the SIM7600 power input pins. This acts as a localized energy reservoir.

#### Stage 3: Logic Level Regulation

- The ESP32-S3 module typically includes an onboard AMS1117-3.3 LDO to drop the 5V system rail to 3.3V for the MCU and NavIC module.

### 4.2 Circuit Interfacing and Logic Levels

The ESP32-S3 operates at 3.3V logic. The SIM7600 often has a logic level of 1.8V but includes level shifters on breakout boards. The NavIC module is 3.3V.

Pin Mapping Table (ESP32-S3):

| Peripheral | ESP32-S3 Pin | Function | Notes |
| --- | --- | --- | --- |
| NavIC TX | GPIO 44 | UART1 RX | Receives NMEA stream. |
| NavIC RX | GPIO 43 | UART1 TX | Sends configuration commands (if needed). |
| SIM7600 TX | GPIO 17 | UART2 RX | Receives AT command responses. |
| SIM7600 RX | GPIO 18 | UART2 TX | Sends AT commands. |
| Camera | DVP Bus | Dedicated | Uses ~14 pins (D0-D7, XCLK, PCLK, VSYNC, HREF, I2C). |
| SD Card | SPI Bus | GPIO 21, 47, etc. | Specific to board layout (e.g., XIAO Sense uses internal SD pins). |

Note on Camera/UART Conflict: The ESP32-S3 has flexible GPIO routing (GPIO Matrix), but physical pin availability is limited on smaller boards like the XIAO. The XIAO ESP32S3 Sense solves this by routing the camera pins internally and exposing the UART/I2C pins on the headers, making it ideal for this multi-peripheral setup.

## 5. Computer Vision Subsystem: OpenCV and AI Implementation

This section details the core differentiation of the system: the automated counting of passengers. While the prompt specifies OpenCV, modern embedded implementation typically uses a hybrid of OpenCV for pre-processing and TensorFlow Lite (TFLite) for robust detection.

### 5.1 The Role of OpenCV on ESP32

OpenCV (Open Source Computer Vision Library) is traditionally a heavy library for desktop processors. However, minimized versions (e.g., esp-dl or lightweight C++ implementations) are used on the ESP32 for:

- Image Pre-processing:

  - Resizing: Converting the raw 1600x1200 capture to the model's input size (e.g., 96x96 or 160x160).

  - Color Space Conversion: Converting RGB565 to Grayscale or RGB888.

  - Normalization: Scaling pixel values from to [0.0, 1.0] or [-128, 127] (signed int8) as required by the quantized neural network.

- Basic Motion Detection (Optional): Using Frame Differencing (subtracting the current frame from the previous one) to trigger the heavy AI inference only when movement is detected. This saves battery and CPU cycles.

### 5.2 Seat Counting Algorithm Options

#### Option A: Classical OpenCV (Blob Detection)

- Mechanism: Background Subtraction. The system captures a "reference" empty bus image. During operation, it subtracts the reference from the current frame. Significant differences (blobs) are counted as people.

- Pros: Fast, low memory usage.

- Cons: Highly sensitive to lighting changes (shadows, sun glare) and requires a fixed background (bus moving creates vibration/shifting). Not recommended for robust deployment.

#### Option B: Deep Learning (Object Detection) - Recommended

- Mechanism: A Convolutional Neural Network (CNN) trained to detect "Person" or "Head" classes.

- Model: FOMO (Faster Objects, More Objects). This is a constrained detection model developed by Edge Impulse. Unlike MobileNet SSD which draws bounding boxes, FOMO divides the image into a grid and classifies each cell. It is extremely fast (~10-30 fps on ESP32-S3) and accurate for counting dense objects.

- Workflow:

  - Data Collection: Capture 1,000+ images of the bus interior under various lighting conditions.

  - Labeling: Annotate images with centroids on passengers' heads.

  - Training: Train the quantized int8 model.

  - Deployment: Export as an Arduino/C++ library.

### 5.3 Implementation Strategy

The firmware will implement Option B for accuracy, using OpenCV functions for the image manipulation pipeline.

/* Conceptual C++ Implementation for Vision Task */
#include <esp_camera.h>
#include <TensorFlowLite_ESP32.h> 

void visionTask(void *pvParameters) {
  while (true) {
    // 1. Capture Frame
    camera_fb_t * fb = esp_camera_fb_get();
    if (!fb) continue;

    // 2. Pre-processing (OpenCV-style operations)
    // Convert JPEG to RGB888 if necessary, or use raw RGB565
    // Resize to 96x96 (Model Input)
    uint8_t *input_tensor = preprocess_image(fb->buf, fb->len);

    // 3. Inference
    interpreter->Invoke();

    // 4. Post-processing
    // Count the number of active cells in the output tensor
    int passenger_count = 0;
    for (int i = 0; i < output_size; i++) {
        if (output_tensor[i] > CONFIDENCE_THRESHOLD) {
            passenger_count++;
        }
    }
    
    // 5. Update Global State
    xSemaphoreTake(telemetryMutex, portMAX_DELAY);
    global_telemetry.occupancy = passenger_count;
    xSemaphoreGive(telemetryMutex);

    esp_camera_fb_return(fb);
    vTaskDelay(pdMS_TO_TICKS(1000)); // Run every second
  }
}

Ref:

## 6. Positioning Subsystem Implementation (NavIC)

The firmware must robustly handle NMEA streams.

### 6.1 NMEA Sentence Analysis

NavIC modules output standard NMEA sentences but with specific Talker IDs. The critical sentences are:

- $GIGGA / $GNGGA: Global Positioning System Fix Data. Contains Latitude, Longitude, Altitude, Fix Quality, and Number of Satellites.

- $GIRMC / $GNRMC: Recommended Minimum Specific GNSS Data. Contains Time, Date, Latitude, Longitude, Speed, and Course.

Parsing Challenge: Many libraries (like standard TinyGPS) hardcode checks for $GPGGA. To support NavIC, the TinyGPS++ library instance must be initialized to accept custom talker IDs or the library header file must be modified to recognize $GI.

### 6.2 Code Implementation Detail

The serial buffer must be read continuously in a background task or loop to prevent buffer overflow (standard UART buffer is 64-128 bytes).

/* NavIC Parsing Logic */
#include <TinyGPS++.h>

TinyGPSPlus navic;
HardwareSerial gpsSerial(1);

void setupGPS() {
  gpsSerial.begin(9600, SERIAL_8N1, 44, 43); // Adjust pins for board
}

void loopGPS() {
  while (gpsSerial.available() > 0) {
    char c = gpsSerial.read();
    navic.encode(c); // TinyGPS++ parses valid sentences regardless of Talker ID
  }
  
  if (navic.location.isUpdated()) {
    // Validate Accuracy
    if (navic.hdop.hdop() < 2.0) { // HDOP < 2.0 implies good precision
       updateLocation(navic.location.lat(), navic.location.lng());
    }
  }
}

Ref:

## 7. Telematics and Communication Subsystem

The SIM7600 modem provides the bridge to the cloud. The firmware interacts with it via AT commands sent over UART.

### 7.1 Connection Workflow

- Initialization: AT (Ping) -> AT+CPIN? (Check SIM) -> AT+CSQ (Check Signal).

- Network Registration: AT+CREG? (Check GSM reg) -> AT+CGREG? (Check GPRS/LTE reg).

- APN Configuration: AT+CGDCONT=1,"IP","<APN_NAME>" (Set APN e.g., 'airtelgprs.com').

- Data Session: AT+CGACT=1,1 (Activate Context).

### 7.2 MQTT Telemetry Upload

MQTT is chosen over HTTP for telemetry due to its lower overhead.

- Library: TinyGSM provides a high-level abstraction for the SIM7600, allowing it to be used with the standard PubSubClient library.

- Topic Design:

  - buses/uplink/{bus_id}: For sending status (Loc, Count, Speed).

  - buses/downlink/{bus_id}: For receiving commands (e.g., "Reset", "Update Config").

### 7.3 HTTP Image Upload (Evidence)

When the system detects a "Full" bus or a panic alert, it can upload an image. This requires an HTTP POST Multipart request.

- AT Command Sequence:

  - AT+HTTPINIT

  - AT+HTTPPARA="URL","http://api.server.com/upload"

  - AT+HTTPDATA=<size>,<time>

  - (Send Raw JPEG Bytes via UART)

  - AT+HTTPACTION=1 (POST)

  - AT+HTTPTERM

- Optimization: The image should be buffered in chunks to avoid overflowing the modem's UART FIFO buffer.

## 8. Backend Infrastructure and Data Visualization

The backend must handle high-velocity data ingestion and provide real-time visualization.

### 8.1 Database Schema (MongoDB)

MongoDB is selected for its flexible schema (BSON), allowing the data structure to evolve without downtime.

Collection: vehicle_telemetry

{
  "_id": ObjectId("..."),
  "device_id": "ESP32-BUS-001",
  "timestamp": ISODate("2023-10-27T10:00:00Z"),
  "location": {
    "type": "Point",
    "coordinates": [77.5946, 12.9716] // [Lng, Lat] for GeoSpatial Indexing
  },
  "metrics": {
    "speed_kmh": 45.2,
    "heading": 180,
    "satellites": 8,
    "hdop": 1.2
  },
  "occupancy": {
    "count": 32,
    "capacity_percentage": 75,
    "confidence_score": 0.85
  },
  "network": {
    "signal_strength": 24, // CSQ
    "technology": "4G"
  }
}

### 8.2 Dashboard and User Interface

- Fleet Manager Dashboard: A Web Application (React.js) using Leaflet or Mapbox GL JS.

  - Real-time Map: Buses appear as moving markers. Marker color indicates occupancy (Green: Low, Orange: Med, Red: High).

  - Geofencing: Alerts when buses deviate from assigned routes.

- Passenger App: A mobile app consuming the same API to show "Live Bus Status" and "Seat Availability", improving the commuter experience.

## 9. Software Architecture and Firmware Design

The complexity of handling networking, vision, and GPS simultaneously necessitates a multi-threaded approach using FreeRTOS.

### 9.1 RTOS Task Decomposition

| Task Name | Priority | Core Affinity | Description |
| --- | --- | --- | --- |
| Task_GPS | High | Core 1 | Continuously reads UART buffer from NavIC. Updates global Location struct protected by a Mutex. |
| Task_Vision | Medium | Core 0 | Captures image, runs TFLite inference. Updates global Occupancy struct. Runs periodically (e.g., every 5s) or on motion trigger. |
| Task_Net | Medium | Core 1 | Manages MQTT connection. Publishes telemetry payload every X seconds. Handles Reconnection logic. |
| Task_Housekeeping | Low | Core 0 | Checks battery voltage, system temperature, and Watchdog timer. |

### 9.2 Memory Management

- PSRAM Usage: The Task_Vision requires significant RAM. The ESP32-S3's PSRAM (SPIRAM) must be enabled in the PlatformIO build_flags (-DBOARD_HAS_PSRAM). The camera frame buffer should be allocated in PSRAM (fb_location = CAMERA_FB_IN_PSRAM).

- Stack Sizing: Tasks running heavy libraries (like SSL/TLS for MQTT) need large stack sizes (e.g., 8KB-12KB) to prevent stack overflow crashes.

## 10. Execution Plan and Roadmap

### Phase 1: Proof of Concept (Weeks 1-4)

- Bench Setup: Wire ESP32-S3, SIM7600, and NavIC module on a breadboard.

- Code Validation: Write separate sketches to validate:

  - NavIC NMEA parsing (verifying $GI sentences).

  - SIM7600 MQTT Connectivity (Pub/Sub).

  - Camera streaming (WebServer example).

### Phase 2: AI Model Development (Weeks 5-8)

- Data Acquisition: Install camera in a bus/mockup. Collect 2,000+ images of people sitting, standing, and empty seats.

- Labeling & Training: Use Edge Impulse to label and train the FOMO object detection model.

- Optimization: Quantize the model to int8 and validate inference time on the ESP32-S3 (Target: < 200ms).

### Phase 3: Hardware Integration (Weeks 9-12)

- PCB Design: Design a custom carrier board integrating the PSU (LM2596), ESP32-S3 socket, and connectors for GSM/GPS antennas.

- Assembly: Solder components and perform electrical testing (Power ripple, Thermal checks).

- Enclosure: Design a ruggedized, heat-dissipating enclosure (ABS/Polycarbonate).

### Phase 4: Field Trials (Weeks 13-16)

- Pilot Deployment: Install in 5 buses on varied routes.

- Data Verification: Compare manual passenger counts with system logs. Tune the model confidence thresholds.

- Dashboard Launch: Release the Beta dashboard to fleet managers.

## 11. Challenges, Mitigation, and Future Advancements

### 11.1 Technical Risks and Mitigations

- Camera Vibration: Bus vibrations can cause image blur. Mitigation: Use a global shutter camera (if budget allows) or fast shutter speed settings on OV2640. Software stabilization in OpenCV pre-processing.

- Variable Lighting: Tunnels and night driving affect vision. Mitigation: Use an IR-cut camera with active IR LED illumination for night vision.

- GSM Dead Zones: Mitigation: Implement a "Store and Forward" mechanism. Data is logged to an SD card (CSV/JSON) and bulk-uploaded when the network returns.

### 11.2 Future Advancements

- Stereo Vision: Upgrading to dual cameras to create a depth map. This allows for distinguishing between humans and luggage based on height, significantly improving counting accuracy (98%+).

- V2X Integration: Enabling the bus to communicate with traffic signals (V2I) to request signal priority (Green Light Extension) when the bus is behind schedule or overcrowded.

- 5G Connectivity: Moving to 5G (SIM8200) to enable real-time high-definition video surveillance for passenger security, beyond just counting.

## 12. Conclusion

The Smart Bus Tracking and Seat Counting System proposed herein represents a technologically advanced, sovereign, and scalable solution for modernizing public transit. By synergizing the indigenous precision of NavIC, the edge-computing prowess of the ESP32-S3, and the ubiquity of LTE connectivity, this architecture addresses the core inefficiencies of fleet management. The shift from simple location tracking to intelligent, vision-based capacity analytics provides operators with the data needed to optimize resources, reduce costs, and ultimately, improve the quality of service for the commuter. The detailed engineering roadmap provided ensures a structured path from prototype to mass deployment, mitigating risks through rigorous hardware selection and robust software architecture.

#### Works cited

1. ESP32 vs ESP32-S3: Key Differences, Performance Comparison, and PCB Design Considerations - PCB Design & Layout - PCBway, https://www.pcbway.com/blog/PCB_Design_Layout/ESP32_vs_ESP32_S3_Key_Differences_Performance_Comparison_and_PCB_Design_Consi_2c205f9a.html 2. ESP32 S3 vs ESP32 : Definition & the Differences - ElProCus, https://www.elprocus.com/difference-between-esp32-s3-vs-esp32/ 3. Edge AI Room Occupancy Sensor with ESP32 and Person Detection - Makerguides.com, https://www.makerguides.com/edge-ai-room-occupancy-sensor-with-esp32-and-person-detection/ 4. XIAO: Big Power, Small Board - 4.4 Image Classification - GitHub Pages, https://mjrovai.github.io/XIAO_Big_Power_Small_Board-ebook/chapter_4-4.html 5. How to Use NavIC GPS: Examples, Pinouts, and Specs - Cirkit Designer Docs, https://docs.cirkitdesigner.com/component/4a7cd1ec-638b-47c2-ae71-b4dcb7cd02e5/navic-gps 6. Dual-Band Multi-System GNSS Positioning Module GEM1205, https://docs.datagnss.com/rtk-board/files/T-5-2206-GEM1205%20Datasheet-V1.3.pdf 7. GNSS L89 R2.0 - Quectel, https://www.quectel.com/product/gnss-l89-r2/ 8. GPS: GGA, RMC, and GLL NMEA sentence structure - TavoTech, https://tavotech.com/gps-nmea-sentence-structure/ 9. Build a Smart ESP32 GPS Tracker with SIM800L and NEO6M Module - Circuit Digest, https://circuitdigest.com/microcontroller-projects/esp32-gps-tracker-with-sim800l-and-neo6m-module 10. How to Use SIM7600 with ESP32 to Send API Requests - Protonest IoT - Medium, https://protonestiot.medium.com/how-to-use-sim7600-with-esp32-to-send-api-requests-8ea6830dabd1 11. limengdu/SeeedStudio-XIAO-ESP32S3-Sense-camera - GitHub, https://github.com/limengdu/SeeedStudio-XIAO-ESP32S3-Sense-camera 12. Automatic Object Counting System with ESP32 Camera & OpenCV | Industrial Product Counter Tutorial - YouTube, https://www.youtube.com/watch?v=jUIBM-Ijzos 13. How to Use LM2596: Examples, Pinouts, and Specs - Cirkit Designer Docs, https://docs.cirkitdesigner.com/component/a8b4f46f-ab87-4f7e-9999-21f2ba787853/lm2596 14. Powering ESP32 with 12V Power Source using Buck Converter - ESPBoards, https://www.espboards.dev/blog/powering-esp32-from-12v-power-source/ 15. ESP32 SIM800L GSM/GPRS Module Pinout, Wiring, ESP32 and more - ESPBoards, https://www.espboards.dev/sensors/sim800l/ 16. ESP32-CAM Person Detection Experiment With TensorFlow Lite : 4 Steps - Instructables, https://www.instructables.com/ESP32-CAM-Person-Detection-Expreiment-With-TensorF/ 17. Detecting empty room - OpenCV Q&A Forum, https://answers.opencv.org/question/5970/detecting-empty-room/ 18. Seeed XIAO ESP32S3 Object Detection - GitHub Pages, https://harvard-edge.github.io/cs249r_book/contents/labs/seeed/xiao_esp32s3/object_detection/object_detection.html 19. XIAO ESP32S3 Sense (Image Classification) - Seeed Studio Wiki, https://wiki.seeedstudio.com/tinyml_course_Image_classification_project/ 20. Library: TinyGPS++ - Codebender, https://codebender.cc/library/TinyGPS++ 21. nmea.c - wertarbyte/tiny-gps - GitHub, https://github.com/wertarbyte/tiny-gps/blob/master/nmea.c 22. ESP32 GPS Module Setup: A Complete Guide for Beginners (2025) | Steve Zafeiriou, https://stevezafeiriou.com/esp32-gps-module-setup/ 23. SIM7600E + ESP32: Unable to Send HTTPS POST to Firebase, HTTP_PEER_CLOSED, https://stackoverflow.com/questions/79745593/sim7600e-esp32-unable-to-send-https-post-to-firebase-http-peer-closed 24. Using image processing to infer if a seat is occupied, https://softwarerecs.stackexchange.com/questions/3333/using-image-processing-to-infer-if-a-seat-is-occupied 25. ESP32-CAM with SIM7600 HTTP POST image - Arduino Forum, https://forum.arduino.cc/t/esp32-cam-with-sim7600-http-post-image/1058837 26. Data Modeling Strategies for Connected Vehicle Signal Data in MongoDB, https://www.mongodb.com/company/blog/innovation/data-modeling-strategies-connected-vehicle-signal-data-in-mongodb 27. Chicago Uses MongoDB To Create A Smarter And Safer City, https://www.mongodb.com/solutions/customer-case-studies/city-of-chicago 28. How to use lm2596 with a esp32 on a pcb - General Electronics - Arduino Forum, https://forum.arduino.cc/t/how-to-use-lm2596-with-a-esp32-on-a-pcb/1350623 29. How to build a simple GPS Tracker using ESP32 and Visualize Data on Map - Circuit Digest, https://circuitdigest.com/microcontroller-projects/simple-gps-tracker-using-esp32-visualize-data-on-map